# OBSETECH T-Core

# Reliable and popular uncooled LWIR shutter thermal imaging core with excellent impact and vibration resistance designed for multiple integrations: vehicles, UGVs, UAVs and other systems. T-Core supports analog and digital video output incl. USB 3.0 and 16bit raw data.

| Detector |  |
| --- | --- |
| Type | Uncooled VOx Microbolometer |
| Pixel resolution | 640×512 or 800x600 (optional) |
| Pixel pitch | 17μm (12μm optional) |
| Spectral Range | 8～14μm |
| Sensitivity (NETD) | ≤40mK @ f/1.0 / 25℃ |
| Image |  |
| Frame rate | 25Hz or 50Hz (PAL), 30Hz or 60Hz (NTSC) |
| Analog video | PAL / NTSC (switchable) |
| Digital video | 8-bit or 14-bit serial LVDS, 8-bit or 14 bit parallel CMOS, 8-bit BT656 (Support BT601, MSB/LSB) |
| Digital zoom | 1x, 2x, 4x |
| Color palettes | 9 standard (customized on demand) |
| Boot time | < 15 s |
| Compensation | Shutter |
| Automatic Gain Control | Yes, adaptive dynamic range compression |
| Intelligent image enhancement | Yes |
| SDK | Support secondary development and function expansion |
| Control |  |
| Communication protocol | RS232-TTL |
| Baud rate | 115200 (default) |
| Power |  |
| Power supply | 4.0 - 6.0 VDC |
| Power consumption | ≦1.8W @ 5V @ 23 ± 3℃ |
| Interface |  |
| Standard external interface | 50-pin Hirose: DF 12-50DS-0.5V(86) |
| Environment |  |
| Operating temperature | -40℃～+70℃ external temp |
| Storage temperature | -45℃～+85℃ external temp |
| Humidity | 5-95% non-condensing |
| Certification | ROHS, REACH and WEEE |
| EMC | FCC, CE Class B |
| Physical Characteristic |  |
| Dimension | 44.5mm x 44.5mm x 44.5mm (W x H x L)（without lens） |
| Weight | < 90g（without lens incl. M34 lens adapter ring） |
| Lens interface | M34/0.5 |
| Mounting interface | 6 attach points in lens mount, M2 x 0.4 x on 3 sides, 2 per side |
| Optional features |  |
| Expansion boards (VPC, Camera Link or USB 3.0 board), Tripod, M34 lens adapter ring, Multiple lens available (athermalized, continuous zoom, manual or motorized focus etc.) |  |

Temperature measurement version available on request. Specific requirements can be customized.