# OBSETECH XU-Core

# Small and reliable uncooled LWIR shutterless thermal imaging core with excellent vibration resistance designed for multiple integrations: UAVs, UGVs, weapon sights, handheld and other systems. XU-Core supports USB 2.0/3.0 raw data output.

| Detector |  |
| --- | --- |
| Type | Uncooled FPA Microbolometer (Ulis) |
| Pixel resolution | 640×480 |
| Pixel pitch | 17μm |
| Spectral Range | 8～14μm |
| NETD | ≤65mk@30℃ |
| Image |  |
| Frame rate | 25Hz or 50Hz |
| Noise reduction | Digital filtering |
| Digital zoom | 2x, 4x, 8x / Smooth zoom |
| Polarity reversal | Available |
| Image flip | Horizontal, Vertical, 180° Flip |
| Startup | ≦3s |
| NUC | NBT shutterless technology, Digital calibration |
| Display |  |
| Video output | CVBS 50Hz（PAL）/ 60Hz（NTSC） |
| Palette | 12 palettes |
| Gain/Brightness adjustment | Auto / Manual |
| Interface |  |
| Serial port | 3.3V TTL |
| Analogue video | 1 port |
| Digital video | BT656 |
| USB | Output RAW, YUV and command, support UVC/CDC |
| Power |  |
| Operating voltage | 5.0 - 12.0 VDC |
| Power consumption | ≦2.0 W |
| Environment |  |
| Operating temperature | -40℃～+65℃ |
| Storage temperature | -50℃～+85℃ |
| Humidity | 5-95% non-condensing |
| Axial shock level | Axial：100g/6ms，GJB 150-18 |
| Lateral shock level | Lateral：40g/11ms，GJB 150-18 |
| Vibration | GJB 150-16 2.3.1 |
| Physical Characteristic |  |
| Dimension | 32mm x 32mm x 41mm (W x H x L)（without lens） |
| Weight | ＜65g（without lens） |

Pin Definition